# MTE 204 – Numerical Methods

# Tutorial #3 Questions

# Question 1:

A manufacturing plant produces three different devices. Three materials (metal, plastic, and rubber) are required to produce each device. The amount of material needed for each device is detailed in the table below

| Device | Metal (g/unit) | Plastic (g/unit) | Rubber (g/unit) |
| --- | --- | --- | --- |
| Device #1 | 15 | 0.25 | 1 |
| Device #2 | 17 | 0.33 | 1.2 |
| Device #3 | 19 | 0.42 | 1.6 |

The plant is consuming 2.12 kg of metal, 0.0434 kg of plastic, and 0.164 kg of rubber daily. Solve for the number of devices produced daily. Use Gaussian elimination with partial pivoting.

# Question 2:

Four springs of differing stiffness are placed in series and a 20 N force is applied to the springs. The following equations are the force balances between each spring, with displacement defined relative to the fixed end of the springs.

k2x2-x1=k1x1
k3x3-x2=k2x2-x1
k4x4-x3=k3x3-x2
F=k4(x4-x3)

Let k1=15 N/cm, k2=5 N/cm, k3=8 N/cm, and k4=23 N/cm.

- Express the equations matrix form.

- Demonstrate that using the Gauss-Seidel method will result in convergence.

- Solve the problem iteratively using the Gauss-Seidel method. Show the system of equations used to perform a Gauss-Seidel iteration. Carry out two iterations of the Gauss-Seidel method, showing you work for the first iteration. Use 10 cm as an initial value for all displacements.